Part 10 – FM Setup:

What is FM?  The Megadrive/Genesis contains a sound chip called the YM2612 (Yamaha Sound Chip), we’ve used this chip already to play the drum samples (DAC), but the chip’s main function is to generate some frequency sounds.  It can make such sounds as a Guitar, Piano, Horns, Violins, and many other sounds, such as strange unique instruments that don’t exist, it has the power to generate a lot of unique and interesting sounds which may sound pleasing to the ear.

So enough about the introduction to FM, lets get to using it.

Firstly, I think it would be a good idea to stop out DAC notes from playing, so let’s do that first:

At offset location “00080100” I’ve put an F2 to stop the DAC channel playing, so now the DAC won’t play while our FM notes are playing (You don’t have to do this if you have some drum notes you want to play with the FM).

Voices:

Now voices are basically instruments, they tell the YM2612 how to make the sound, I won’t go into details about editing voices or making them, as that would call for another tutorial or guide, but you can visit the SCHG SMPS music guide on Sonic Retro for more info on voices.

The problem is before using FM; we need voices, so where can we get them?  There are many ways:

(A) Make them yourself, though it’s not recommended if you don’t have experience.

(B) Collect them from other games; however that would mean finding them.

(C) Collect them from sonic 1 (The game we’re hacking), this would me more appropriate.

We’re gonna follow (C), simply because it’s the easiest.  But if you wish to use other voices, then checkout http://sonicsanctuary.digibase.ca/ for more information, the site contains a shit load of voices which you might find useful.

So where shall we get our voices?  Let’s get ours from GHZ, goto offset location “74C63”, you’ll get something like this:

OK, doesn’t look like much, let me just highlight it for you:

That’s better, now you can see that I’ve highlighted each small section with a colour, every 25 ($19 in Hex) bytes is one voice, I’ll explain what each one is:

00 Red: = Base Guitar Version 01

01 Green = Base Guitar Version 02

02 Blue = Bell

03 Purple = Horn/Violin

04 Yellow = Lead Version 01

05 Light Blue = Lead Version 02

06 Pink = Bell Piano

07 Grey = Horn

08 Orange = Lead Version 03

Now admittedly I’m 100% sure that’s what those voices are and I’m just going by what sound they make.

We’re gonna be using these voices, so, copy all those voices (The ones highlighted in the example image), and then go down to offset location “00080000” where our music is.  Now we need a place to put these voices, but it needs to be a sensible place.  I’m gonna recommend offset location “00080200”, that’ll do, a nice empty place, like so:

Next we need to change the voice pointer in the header at “00080000”; we set it to “CCCC” temporarily because we didn’t have any voices at the time.

So, let’s set the pointer:

“00080200” – “00080000” = “00000200”

“0200” is your answer, so put that in the header:

The voices are now ready to be used, next up we need to get some FM notes in, so let’s find a place to put our FM notes, I will recommend offset “00080300” simply because there’s a lot of empty space after it that’s unused, so change the FM 1 pointer to “0300” so it points to “00080300”:

And there we are, just go down to offset “00080300” and we can start getting some FM notes in.

Part 11 – The EF Flag:

Making music now, you can see at “00080300” I’ve put an “EF00” in to begin with.  Back at “Part 08 – Co-ordination Flag List”, you’ll see:

| EF | Voice Changer Flag | Sonic 1/Sonic 3K | Used to change the FM voice being used |
| --- | --- | --- | --- |

This is the flag we’re using, and it’s set-out like so “EF ??”, the “??” is what voice to use, if you remember in the last part we copied the voices from GHZ, and they were listed in order of:

00 Red = Base Guitar Version 01

01 Green = Base Guitar Version 02

02 Blue = Bell

03 Purple = Horn/Violin

04 Yellow = Lead Version 01

05 Light Blue = Lead Version 02

06 Pink = Bell Piano

07 Grey = Horn

08 Orange = Lead Version 03

If we put EF 02, the FM channel will use the Bell voice.

If we put EF 01, the FM channel will use the Base Guitar Version 02 voice

If we put EF 06, the FM channel will use the Bell Piano voice

In this case, I’ve used “EF 00”, this means it’s using the first voice in the list (Base Guitar Version 01), and that’s all there is to this flag.

Part 12 – FM Notes:

Now for the actual notes, to place a note, you need to put a value from 80 to DF, the way it works is; 80 is a rest, while 81 to DF is from the lowest note to the highest note, here’s a list below to help you understand:

[As documented by the “SCHG SMPS Music” guide]

80 = Rest

81 = 0C

82 = 0C♯ or 0D♭

83 = 0D

84 = 0D♯ or 0E♭

85 = 0E

86 = 0F

87 = 0F♯ or 0G♭

88 = 0G

89 = 0G♯ or 0A♭

8A = 0A

8B = 0A♯ or 0B♭

8C = 0B

8D = 1C

8E = 1C♯ or 1D♭

8F = 1D

90 = 1D♯ or 1E♭

91 = 1E

92 = 1F

93 = 1F♯ or 1G♭

94 = 1G

95 = 1G♯ or 1A♭

96 = 1A

97 = 1A♯ or 1B♭

98 = 1B

99 = 2C

9A = 2C♯ or 2D♭

9B = 2D

9C = 2D♯ or 2E♭

9D = 2E

9E = 2F

9F = 2F♯ or 2G♭

A0 = 2G

A1 = 2G♯ or 2A♭

A2 = 2A

A3 = 2A♯ or 2B♭

A4 = 2B

A5 = 3C

A6 = 3C♯ or 3D♭

A7 = 3D

A8 = 3D♯ or 3E♭

A9 = 3E

AA = 3F

AB = 3F♯ or 3G♭

AC = 3G

AD = 3G♯ or 3A♭

AE = 3A

AF = 3A♯ or 3B♭

B0 = 3B

B1 = 4C

B2 = 4C♯ or 4D♭

B3 = 4D

B4 = 4D♯ or 4E♭

B5 = 4E

B6 = 4F

B7 = 4F♯ or 4G♭

B8 = 4G

B9 = 4G♯ or 4A♭

BA = 4A

BB = 4A♯ or 4B♭

BC = 4B

BD = 5C

BE = 5C♯ or 5D♭

BF = 5D

C0 = 5D♯ or 5E♭

C1 = 5E

C2 = 5F

C3 = 5F♯ or 5G♭

C4 = 5G

C5 = 5G♯ or 5A♭

C6 = 5A

C7 = 5A♯ or 5B♭

C8 = 5B

C9 = 6C

CA = 6C♯ or 6D♭

CB = 6D

CC = 6D♯ or 6E♭

CD = 6E

CE = 6F

CF = 6F♯ or 6G♭

D0 = 6G

D1 = 6G♯ or 6A♭

D2 = 6A

D3 = 6A♯ or 6B♭

D4 = 6B

D5 = 7C

D6 = 7C♯ or 7D♭

D7 = 7D

D8 = 7D♯ or 7E♭

D9 = 7E

DA = 7F

DB = 7F♯ or 7G♭

DC = 7G

DD = 7G♯ or 7A♭

DE = 7A

DF = 7A♯ or 7B♭

Right then, now we have a list, let’s get started with an example:

I’ve made a simple one here for you to test out, save and play it. It works in this order:

EF 00 = Select first voice in list

AE = Note 3A

40 = set to wait 40 cycles

F2 = Stop

So basically, it sets the voice to Base Guitar Version 00, plays note 3A, waits 40 cycles, then stops the channel.

That should be self explanatory, but then again, having only 1 note play is a little boring, let’s renovate it up a bit, how about this:

EF 00 = Select first voice in list

AE = Note 3A

08 = set to wait 08 cycles

BA = Note 4A

04 = set to wait 04 cycles

AE = Note 3A

AF = Note 3A♯ or 3B♭

08 = set to wait 08 cycles

BB = Note 4A♯ or 4B♭

04 = set to wait 04 cycles

AF = Note 3A♯ or 3B♭

B0 = Note 3B

08 = set to wait 08 cycles

BC = Note 4B

04 = set to wait 04 cycles

B0 = Note 3B

B1 = Note 3C

08 = set to wait 08 cycles

BD = Note 4C

04 = set to wait 04 cycles

B1 = Note 3C

F2 = Stop

Give that a try, sounds kind of funky huh?  But there’s just something not right about it, firstly it’s a bit too loud, and it’s also too high in pitch, let’s sort it out.  Now I said we’d come back to this, so we are now, go back up to offset “00080000” where our header is:

You’ll notice that next the FM1’s pointer I’ve highlighted two bytes.  The left byte is the pitch, and the right byte is the volume.

The Volume:

00 is the highest/loudest volume you can have, the highest that value is; the quieter the channel will play.  The quietest value depends on the voice generally, but an average is from 40 to 50 is generally the most silent.

We’ll be changing ours to about 08, that should be quiet enough but not so quiet that we can’t hear it:

The Pitch:

00 is the default pitch, if you increase it to 01, the pitch is raised by 1 note, and if you increase it by 02, the pitch is raised by 2 notes, you can increase it to 7F the maximum, although the chances are you’ll never increase it to that level

You can decrease it by 1 setting it at FF, that’ll lower the pitch by 1 note, if you decrease it by 2 setting it at FE, that’ll lower the pitch by 2 notes, you can decrease it down to 80 the lowest, although again, the chances are, you’ll never decrease it that low.

We’re gonna change ours so to F4, so that it’s low enough to sound like a proper base:

So take a listen, I think that’s fixed it.  Though, maybe we could change the voice to the second Base Guitar Version, but this is just a personal choice, you’ll be making a lot of these when making music yourself, so you don’t have to change it if you don’t want to, but I will be in this tutorial:

There, I think that sounds better.

This is just a general example, so once you’ve had a listen and understand what they do, go back in the header at offset “00080000”, I think we can change the pitch back to 00, but leave the volume at 08 for this next part of the tutorial:

Part 13 – Co-ordination Flags (FM):

Did you know, you can use flags that we used for the DAC?  That’s right; you can use F6, F7, F8, E3, E0, and F2 for FM channels too!  And they’re all the same rules.

In this section we’re gonna be looking at newer flags which haven’t been explained yet, these flags can be useful under certain circumstances.

EF - Voice Changer Flag:

“[Used to change the FM voice being used]”

I think by now you most likely already understand what this flag does, but I’ll re-explain it anyway to make sure.

The EF flag is used to set a channel to play one of the voices in the voice list, here’s an example:

So what happens here is the first EF 01 flag selects to play the second voice (Base Guitar Version 02) in the list:

00 = Base Guitar Version 01

01 = Base Guitar Version 02

02 = Bell

03 = Horn/Violin

04 = Lead Version 01

05 = Lead Version 02

06 = Bell Piano

07 = Horn

08 = Lead Version 03

And then plays four notes “B0”, “B1”, “B2” and then “B1” in the sound of the “Base Guitar”, then it gets to EF 02 and selects the third voice (Bell) in the list, and plays another four notes “B0”, “B1”, “B2” and then “B1” again but in the sound of a “Bell”, then stops because of the F2 stop flag.

And that’s it, you can change what voice it uses in the list at any time in the SMPS notes, so you can have the channel play a base guitar as part of a base, then quickly change to play a melodically bell sound then switch back to play the base again if you wanted, and that’s just one example of many possibilities.

E6 - Channel Volume Changer Flag & EC - Channel Volume Changer Flag (Non-Update):

“[Used to change the volume of the channel]” &

“[Used to change the volume of the channel (without updating)]”

These two flags are similar, which is why they’re explained together here.

The flags are both set-out like “E6 ??” of which the “??” is what to add to the volume.  Now we set our volume to 08 be default in the header, so let’s look at how this works:

In the example, it’ll play 4 notes in a Base Guitar voice, then “E6 08” will add 08 to the volume, our volume is already at 08, so in hex:

08 + 08 = 10

The volume is now 10, and it’s now more silent than before.  Next it plays the same 4 notes again, but this time it’s more quite because the volume was lowered by 08, then another “E6 08” adds another 08 to the volume, in hex:

10 + 08 = 18

The volume is now 18, and it’s even more silent than before.  It plays the same 4 notes again, then “E6 F8” adds F8 to the volume, so in hex:

18 + F8 = 0110 (We’re only interested in the end byte “10”) so = 10

The volume is now 10, which makes it a bit louder than last time.  Then it plays the same 4 notes yet again, and then another “E6 F8” adds F8 to the volume again, like so:

10 + F8 = 0108 (We’re only interested in the end byte “08”) so = 08

The volume is now back to 08, and it plays the last 4 notes again but a bit more loudly, then stops because of the F2 stop flag.  And that’s about it.

Now the same rules apply to the EC flag, except one thing that differs it, this one doesn’t update itself, so what do I mean by that?

Well, when the E6 flag changes the volume, when the next note plays, you notice it strait away, because it updated itself.  The EC flag doesn’t update, which means you won’t hear a volume change until another flag is set, like the EF flag, or the E0 flag.  Because those flags update everything, it’ll update EC’s volume change.  And that’s the only difference.

E9 - Pitch Changer Flag:

“[Used to increase or decrease the pitch by a full note amount.]”

This is used to change the pitch of the channel, so we can play some notices higher or lower in pitch, and is set-out as “E9 ??” of which the “??” is how much to raise or lower the pitch by.  But like all flags in this tutorial, it calls for another example:

In this example we have the note B0 (note 3B) playing, then after 20 cycles of waiting, it hits an “E9 01”, this raises the pitch of the channel by 01, The pitch of our channel was set at 00, so after hitting this flag, in hex:

00 + 01 = 01

Now the pitch is at 01, so when it plays the next note “B0” (note 3B), that note will play as “B1” (note 3C) which is the next note higher. When it gets to “E9 FF”, it’ll lower the pitch, because in hex:

01 + FF = 0100 (We’re only interested in the end byte “00”), so = 00

Now the pitch is at 00 again, so the notes play at the normal pitch.

E5 - Tempo (Channel) Changer Flag,

EA - Tempo (1st & 2nd Byte) Changer Flag &

EB - Tempo (All Channels) Changer Flag:

“[Used to change the channel’s tempo/speed]”,

“[Used to change the header’s word of tempo/speed (First and Second bytes)]” &

“[Used to change every channel’s tempo/speed]”

All of these three are responsible for changing the Tempo of the music, but they are all different in the way that they work, I personally think these are a little weird, but I’ll go ahead and explain anyway.

Firstly to note:

Every channel has two bytes for tempo, a 1st byte (Tempo Divider), and a 2nd byte (The Tempo).  And it’s the same as the header’s 2 bytes.

E5 flag:  This one will change the 2nd byte of the channel’s tempo, so if you set E5 02, it’ll set the channel’s tempo to 02.

EA flag:  This one changes the “Header’s” tempo/speed, but it sets both the 1st and 2nd bytes, so for example if our header is “0103” and you set EB 02, then the header’s tempo will change to “0202”.

(I’m not sure if the EA flag setting both bytes; is a bug in the SMPS engine, or not, but I’ll see into researching this at a later date).

EB flag:  This one does exactly what the E5 flag does, but it does this for ALL channels including DAC, and PSG.

Side note:

These can also be used on the DAC channel; I just forgot to explain it when teaching the DAC part.

E7 - Attack Prevent Flag:

“[Used to stop a note from attacking, so a note can continue or change pitch without re-hitting the note.]”

This flag is pretty useful in situations, but I’m not sure I can explain this well without an example, so:

In this case:

EF 01 = select second voice in list (Base Guitar Version 02)

B0 = note 3B

10 = set to wait 10 cycles

B1 = note 3C

B2 = note 3C♯ or 3D♭

B1 = note 3C

B0 = note 3B

E7 B1 = continue note B0 but in the pitch of B1.

B2 = note 3C♯ or 3D♭

E7 B1 = continue note B2 but in the pitch of B1.

F2 = stop

Basically the only way I can describe it, is by saying it prevents a note from attacking, but if you don’t understand then have a listen to the example for yourself, that should clear things up for you.

E8 – Note Length Flag:

“[Used to set how long a note can play the sound before it stops]”

This is set-out like so “E8 ??” of which the “??” is how long the note plays for, in this example below:

If you play this, you’ll hear the Base Guitar play 3 times normally, then after its hit the “E8 08”, it’ll play another 3 notes, but this time, although the speed of the music remains the same, the notes only hit and play for 08 cycles, then stop while waiting for the next note.

The lower the value is, the shorter in length the note will play for.

To turn off the effect when you don’t need it, set “E8 00”, this will stop the length flag from running.

Checklist:

E0 = done

E1 = not yet

E2 = wont be learnt

E3 = done

E4 = done

E5 = done

E6 = done

E7 = done

E8 = done

E9 = done

EA = done

EB = done

EC = done

ED = wont be learnt

EE = wont be learnt

EF = done

F0 = not yet

F1 = not yet

F2 = done

F3 = not yet

F4 = not yet

F5 = not yet

F6 = done

F7 = done

F8 = done

OK, we’ve nearly got through the lot now, about 5 or 6 more things then you’ll have understood most of the SMPS format.

F0 - Modulation Flag:

“[Used to increase or decrease the pitch simultaneously in a zigzag form (often referred to as pitch “bends”)]”

This is a very favoured flag for its unique use in SMPS music, this flag can cause the pitch to raise then lower simultaneously at a long or quick rate, often known as “Pitch Bends”, however funny enough, they’re not actually “Pitch Bends”, but rather “Pitch Zigzags” as shown in the graph example below:

In the graph there are 4 things which make the modulation.

Delay:

The time it takes before the modulation starts, as you can see in the graph, the green line stays strait for about 6 dots.

Speed:

The graph has to be moving at a speed, unfortunately you cannot see it.  But let’s say that every dot is a millisecond.

Step Size:

A “Step” is the size between two dots, and the size is how large each step should rise or lower in pitch, as seen in the graph after the delay line, the steps rise & lower by 2 spaces (2 grey squares) before playing.

Number Of Steps:

Each step is a dot and in the case of the graph, it takes 2 dots until it gets from centre to the top, 2 to get from top to centre, 2 to get from centre to bottom, etc.

And those four are important for this flag, as the set-out is “F0 ?? ?? ?? ??” where each “??” byte represents one of those four important categories above.

The first “??” byte is the “Delay” amount.

The second “??” byte is the “Speed”.

The third “??” byte is the “Step Size”.

The fourth “??” byte us the “Number Of Steps”.

And that’s it, and if that graph were to be in the flag’s format, it would be “F0 06 01 02 02”, as it waits 6 dots before starting, at 01 speed (Fastest), 2 sizes per step, and 2 steps to move before reversing.

Now what tutorial would this be without a playable example?

In this example the first note plays as normal, but the second note will play normally for 10 cycles, then at the fast speed of 01 it’ll increase in pitch by 08, and will do it 10 times, then reverse the other direction.

Of course this happens very quickly, and you’ll have to listen and play around with it to understand and take full advantage of this flag.

F4 - Modulation Off Flag & F1 - Modulation On Flag:

“[Used to stop Modulation running when it’s turned on]” &

“[Used to run Modulation when it’s turned off]”

OK, so you have just made some modulation and a few notes are playing this modulation, but you want to stop the modulation from running a little while?

These are the flags you need to use, here’s an example:

OK, this is a rather more complex track, but I’ll go ahead and explain anyway:

And there you have it, F4 to stop modulation, F1 to restart it again.  An important note would be that when modulation starts, it automatically turns on modulation, so you don’t need to use F1 to turn it on unless an F4 has turned it off.

Here the example selects the 2nd voice (EF 01) then sets the Modulation to “F0 00 01 30 02” which will make the notes sound wobbly.

It starts playing a base sort of tune twice in modulation, then it selects the 7th voice (EF 06) and then “F4” turns the modulation off.

It plays a melody tune twice but with no modulation, so it doesn’t sound wobbly. Then after selects the 2nd voice (EF 01) and then “F1” turns the modulation back on, and the F6 flag jumps back to offset “00080307” where the first base note is “A0”.

And it’s all repeated endlessly.

For this next flag we need to have two FM channels running, so at offset location “00080000” we need to change the FM2 pointer to point to offset “00080400” as it’s a nice round number:

And now we have two FM channels running at the same time, which we need to control:

Offset “00080300”

Offset “00080400”

E1 – Pitch Tuning Flag:

“[Used to increase or decrease the pitch by a small fraction (Hardly noticeable amount)]”

This flag is set-out “E1 ??” of which the “??” is how much to raise or lower the pitch by, now the thing with this flag is that you will BARELY NOTICE the change.

So why use this flag?  Well there is a special technique which is often used, and even I use it a lot of the time to create some very unique sounds, but how?  Like so:

FM1:

FM2:

Save and play that.  You can hear both channels playing the same exact note, but it just doesn’t sound great, it sounds pretty weird compared to having only 1 channel play it, that’s where the E1 flag comes in:

You’ll notice on my FM2 I’ve put an “E1 02” just before the note plays, this will lift the pitch up by 2 fractions, if you play it now, you’ll hear the results have improved.

You can try maybe raising it to E1 04 or even E1 08 to differ the channels more.

And there you have it, the E1 flag explained.

The idea behind this is that by having the exact same sound generate twice but the second sound raised by a small fraction of pitch, the sound becomes strange and more wave like, it’s hard to explain without becoming all scientific about it.

Checklist:

E0 = done

E1 = done

E2 = wont be learnt

E3 = done

E4 = done

E5 = done

E6 = done

E7 = done

E8 = done

E9 = done

EA = done

EB = done

EC = done

ED = wont be learnt

EE = wont be learnt

EF = done

F0 = done

F1 = done

F2 = done

F3 = not yet

F4 = done

F5 = not yet

F6 = done

F7 = done

F8 = done

We’ve done everything that FM and DAC can use, but now we have two more left, and this is PSG related which is the next thing we’re gonna look at.